# COM 801 Project Elective

PROJECT REPORT

Data Analysis and RAG

Recommendation System

For Compost Monitoring System

Instructor:

Prof. Jyotsna Bapat

and

Prof. Sasirekha GVK

By:

C Siva Kumar

MT2024514

# 1. Introduction

Objective
Our project focuses on analyzing sensor data from two composting cycles to understand the composting process and build a Retrieval-Augmented Generation (RAG) system for the Compost Monitoring System. The goal is to provide actionable recommendations for optimizing composting efficiency, reducing harmful emissions (NH₃, H₂S, CH₄), and improving compost quality.

## Scope

- Data Analysis:

  - Collected and analyzed sensor data from two composting cycles:

    - Cycle 1: Baseline composting process (40+ days).

    - Cycle 2: Improved composting setup with enhanced aeration (40+ days).

- RAG System: Developed a prototype system to retrieve contextually relevant composting guidelines from research papers and generate recommendations.

## Parameters Monitored

- Gases: NH₃, H₂S, CH₄ (emissions control).

- Environmental Factors: Soil moisture, ambient temperature, humidity.

- Compost Health: Compost temperature, pH value.

Fig. 1 Composting Bin Setup

# 2. Part A: Data Analysis

## 2.1 Data Processing

Data Collection
Sensor data was collected wirelessly using a Raspberry Pi. Daily readings were converted into Excel files via a Python script (this was done by Arun Sir) and later standardized into CSV files.

### Preprocessing

- Missing Values: Applied interpolation methods:

| Parameter | Method | Reasoning |
| --- | --- | --- |
| NH₃, Soil Moisture | Linear | Stable trends over time. |
| H₂S | Cubic Spline | Captured non-linear fluctuations. |
| CH₄, pH | Polynomial (Order 2) | Modelled gradual changes in emissions. |
| Humidity | Linear + Rolling Mean | Smoothed short-term noise. |

- Outlier Removal: Used the Interquartile Range (IQR) method. It is also known as Box-Whisker plot.

## 2.2 Data Aggregation

Aggregated daily sensor readings into two datasets:

- Cycle 1: Baseline composting process.

- Cycle 2: Improved setup with better aeration.
Calculated daily averages for each parameter to reduce noise.

Interpolation methods why and what : https://g.co/gemini/share/407d5c7ab2e1

Full A to Z Data analysis: https://chatgpt.com/share/68a8a738-2430-8012-976a-5df19d221743

## 2.3 Data Visualization and Insights

Tools: Pandas, Matplotlib, NumPy.

Fig 2. Cycle 1

Fig 3. Cycle 2

### Key Findings

- Cycle 1:

  - NH₃ and CH₄ correlation: 0.83 (indicating anaerobic conditions).

  - Stalled microbial activity.

  - Insight: Poor aeration slowed decomposition.

- Cycle 2:

  - Compost temperature stabilized at ~30°C by Day 30 (25% faster than Cycle 1).

  - Reduced CH₄ and H₂S spikes.

  - Insight: Improved aeration reduced the anaerobic conditions.

# 3. Part B: RAG-Based Recommendation System

Fig 4. Level 1 diagram of the RAG System

## 3.1 Data Sources

Scraped composting guidelines from PDFs of research papers related to compost monitoring systems, composting practices etc. stored in Google Drive.

## 3.2 Data Processing Pipeline

1. Text Extraction
Used pdfplumber python library to preserve document structure.

2. Cleaning

- Removed citations (e.g., "[3]") and URLs.

- Retained section headers using regex.

3. Chunking
Split text into 1,000-character chunks with 200-character overlap using Recursive Character Text Splitter.

4. Embedding
Generated embeddings with BAAI/bge-small-en-v1.5 model.

Fig 5. Preprocessing Pipeline

5. Vector Database
Why Chroma?

- Chose ChromaDB for prototyping simplicity:

  - Stores embeddings + metadata (e.g., section titles).

  - Enables fast similarity searches.

  - Persists data to disk for reuse.

Fig. 6. Creating Embeddings

## 3.3 Current System Output

- Functional ChromaDB database with 1,200+ text chunks.

- Metadata tags for filtering (e.g., "Aeration", "pH Control").

- Sample query output for retrieving relevant composting guidelines.

# 4. Conclusion

Data Analysis Insights
Cycle 2 demonstrated a 25% faster composting time with improved aeration, validating its role in emission reduction.

RAG System Progress
Built an end-to-end pipeline for knowledge retrieval.

Next Steps:

- Integrate LLMs (e.g. GPT-4 or pen source ones like Llama) for natural language recommendations.

- Link sensor data trends to real-time advice (e.g., pH adjustments).

- Scale with MongoDB/Milvus/Qdrant for production.

Final Notes
This project combines data-driven composting analysis with AI-powered recommendations, offering a sustainable waste management blueprint.